CricRelay

Architecture Decision Record

Technical Build Specification — v1.2 — May 2026

This document describes every hardware component, software service, data flow, security mechanism, and build decision behind the CricRelay grassroots cricket streaming system. Written so that a developer with general Python and Raspberry Pi experience can pick this up and build the full system from scratch.

| Field | Value |
| --- | --- |
| Document ID | ADR-001 |
| Status | Accepted — Active |
| Author | CricRelay Engineering |
| Date | May 2026 |
| Applies to | CricRelay v1.2 — UK market (PCS / PCS Pro) |
| Last reviewed | May 2026 |

# 1. Context and problem statement

CricRelay is a grassroots cricket live streaming system. It must take a 4K video feed from a PTZ camera mounted on a 6-metre mast at the cricket boundary, overlay a live score received from a scorer's phone or laptop, and push a professional-quality stream to YouTube Live — all without any laptop, HDMI cables, or manual camera operation on match day.

The two core problems this architecture solves are:

- Score data is produced on the scorer's device (phone/tablet running PCS, or laptop running PCS Pro) at the scorer's table, which is typically 40–80 metres away from the camera mast. Standard Bluetooth has a range of only 10–30 metres in open air — too short to bridge directly.

- Multiple matches can run simultaneously on adjacent pitches at the same venue. Score data, WiFi networks, and Bluetooth connections must be completely isolated so that Match A's scores never contaminate Match B's stream.

# 2. System overview — three physical devices

The CricRelay system consists of three hardware units that work together. Understanding what each one does and where it physically sits is the foundation for understanding every decision that follows.

| Device | What it is | Where it sits | What it does |
| --- | --- | --- | --- |
| PTZ Camera | Sony IMX415 IP67 outdoor camera | Top of 6m mast at boundary edge | Records 4K video and sends it down the mast via a single ethernet cable |
| CamBox | Raspberry Pi 5 in weatherproof case | Base of 6m mast at boundary | Receives video, adds score overlay, encodes, streams to YouTube via 4G |
| ScorePuck | Raspberry Pi Zero 2W in small case | Scorer’s table (pavilion or boundary) | Receives score data from scorer’s device via Bluetooth or USB, relays it to the CamBox over WiFi |

Think of it this way: the PTZ Camera is the eyes, the CamBox is the brain, and the ScorePuck is the ears that listen to the scorer and whisper the score to the brain across the ground.

# 3. Hardware — component specifications

## 3.1 PTZ Camera

PTZ stands for Pan-Tilt-Zoom. This camera can rotate left/right (pan), tilt up/down, and zoom in/out remotely, all under software control from the CamBox.

| Specification | Value | Why it matters |
| --- | --- | --- |
| Sensor | Sony IMX415 STARVIS 2, 1/1.8" | Large sensor = better colour in overcast UK skies |
| Resolution | 4K @ 30fps / 1080p @ 60fps | 4K future-proofs; 1080p used for streaming to reduce Pi CPU load |
| Optical zoom | 30x (4.7–94mm lens) | Covers 90m from boundary to non-striker’s end clearly |
| IP rating | IP67 | Fully weatherproof — no additional housing or rain cover needed |
| Power | PoE (802.3at, 48V) | A single ethernet cable from the CamBox carries both power AND video up the mast. No separate power cable. |
| Video output | RTSP stream over ethernet | Camera sends video as an IP network stream — no HDMI, no capture card needed |
| PTZ control | VISCA over IP | CamBox sends pan/tilt/zoom commands to camera over the same ethernet cable |
| Supplier | Shenzhen Anxinshi Technology (OEM) | OEM from 50 units, 7–15 day lead time |

PoE (Power over Ethernet) is the key to the single-cable design. The CamBox has a PoE injector that pushes 48V DC power into the ethernet cable alongside the data signal. The camera receives both from the same cable. This is the same technology used in WiFi access points and IP security cameras worldwide.

## 3.2 CamBox (Raspberry Pi 5)

The CamBox is a Raspberry Pi 5 computer housed in a weatherproof IP65 ABS plastic enclosure. It mounts at the base of the mast and does all the heavy lifting: receiving video from the camera, compositing the score overlay, encoding the stream, and pushing it to YouTube.

| Component | Spec | Purpose |
| --- | --- | --- |
| Compute board | Raspberry Pi 5 (4GB RAM) | Main processor. Handles FFmpeg video processing, score overlay rendering, PTZ control, and WiFi hotspot simultaneously |
| Operating system | CricRelay OS (custom Raspberry Pi OS image) | Boots automatically, starts all services, connects to camera and ScorePuck without manual intervention |
| 4G modem | Quectel EC25 USB modem (multi-band UK/AU) | Provides internet for YouTube streaming at any ground. Plugs into Pi USB port. |
| PoE injector | 802.3at 48V injector (passive component) | Sits between the Pi’s ethernet port and the camera cable. Adds 48V power to the cable going up the mast. |
| WiFi | Pi 5 built-in WiFi (802.11ac) | Creates a local WiFi hotspot that the ScorePuck connects to for score relay. Does NOT use this for YouTube — that goes via 4G. |
| Enclosure | IP65 ABS case with pole-mount bracket | Weatherproof. Mounts directly on the mast pole. |
| Status LED | RGB LED wired to Pi GPIO | Green = streaming live. Amber = connecting. Red = fault. Visible from 20m. |
| Power input | USB-C, 5V from power bank | 26,800mAh power bank in the carry case powers the Pi for a full match day (7–8 hours). |

The Pi 5 runs two network connections simultaneously: the 4G modem (for YouTube streaming, which needs internet) and its own WiFi hotspot (for the ScorePuck connection, which is local only). These do not interfere with each other — they are separate network interfaces.

## 3.3 ScorePuck (Raspberry Pi Zero 2W)

The ScorePuck is a small self-contained device in a compact branded enclosure. It sits at the scorer’s table and acts as a relay: it receives score data from the scorer’s device on one side, and sends it to the CamBox over WiFi on the other side. It includes a built-in rechargeable battery providing 10–12 hours of runtime from a single overnight charge — no cables required on match day.

| Component | Spec | Purpose |
| --- | --- | --- |
| Compute board | Raspberry Pi Zero 2W | Quad-core ARM, 512MB RAM. Low power, built-in Bluetooth 5.0 and WiFi. |
| Bluetooth | Built-in BLE 5.0 | Connects to scorer’s PCS app (phone/tablet) and receives score strings ball by ball |
| USB port | Micro-USB OTG (On-The-Go) | When scorer uses PCS Pro on a laptop, a USB cable from laptop to ScorePuck carries score data instead of Bluetooth. Also used for firmware updates. |
| WiFi | Built-in 802.11n | Connects to CamBox’s local hotspot and sends score strings as UDP packets |
| Battery HAT | PiJuice Zero — stacks directly onto Pi Zero 2W | Manages battery charging and power delivery. Provides clean 5V regulated power to Pi at all times. Monitors battery level and triggers low-battery LED warning at 15%. |
| Battery cell | LiPo 3,000mAh, 3.7V, JST connector, 65mm × 50mm × 6mm | 10–12 hours runtime at 250mA average draw. Fits inside enclosure alongside the stacked Pi + PiJuice. |
| Charge controller | Built into PiJuice Zero (TP4056-based) | Charges LiPo safely from USB-C input. Prevents overcharge (stops at 4.2V) and over-discharge (cuts at 3.0V). Charges at 500mA — full charge in ~6 hours overnight. |
| Boost converter | Built into PiJuice Zero (MT3608-based) | Steps LiPo’s 3.7V up to clean 5V for the Pi Zero 2W. 90% efficient. Maintains stable 5V even as battery discharges from 4.2V to 3.0V. |
| Charging port | USB-C, flush-mounted on enclosure end | Single cable overnight charge. Can also power ScorePuck directly from mains or laptop while charging battery simultaneously. |
| Status LED | Single RGB LED | Blue = paired to PCS app via Bluetooth. Green = connected to CamBox. Amber = battery below 15%. Red = fault. |
| Enclosure | 85mm × 55mm × 30mm branded ABS case | Slightly deeper than v1 to accommodate PiJuice HAT stack and LiPo pouch. Still pocket-sized. |

Plain English: The PiJuice Zero is a ready-made battery management board that clips on top of the Pi Zero 2W like a Lego brick. It handles all the complex charging and voltage conversion automatically. The scorer charges it overnight via USB-C (like a phone), puts it on the table at the start of play, and the ScorePuck runs all day without any cables or power source at the ground.

### ScorePuck power budget

| State | Current draw | Power | Notes |
| --- | --- | --- | --- |
| Idle (waiting for BLE) | ~120mA at 5V | 0.6W | Most of the time between balls |
| Active (BLE receive + WiFi transmit) | ~350mA at 5V | 1.75W | Brief burst after each ball entry |
| Average over full match day | ~200–250mA at 5V | ~1.1W | Weighted average across idle and active states |
| Estimated runtime (3,000mAh LiPo) | 10–12 hours | — | At 250mA average, accounting for boost converter losses |
| Charge time (USB-C, 500mA input) | ~6 hours | — | Overnight charge from empty to full |

## 3.4 Physical cable diagram

Here is every physical cable and connection in the complete system:

BOUNDARY MAST (6 metres tall)

[PTZ Camera, IP67, top of mast]

|

| Single ethernet cable (Cat5e, outdoor rated)

| Carries: power up (48V DC via PoE)

|          video down (RTSP stream, IP packets)

|          PTZ commands down (VISCA over IP)

|

[PoE Injector, inside CamBox enclosure]

|

[Raspberry Pi 5 (CamBox)]

|-- USB-A port --> [Quectel 4G USB modem] --> [SIM card] --> Internet

|-- USB-C port <-- [26,800mAh Power Bank]

|-- Built-in WiFi --> local hotspot 'CC-A7F2' (ScorePuck connects here)

|-- Built-in ethernet <-- PoE injector <-- camera cable (above)

~~~~~~~~~~~~~~~~~~~~~  80 metres of open air  ~~~~~~~~~~~~~~~~~~~~~

[ScorePuck (Pi Zero 2W + PiJuice Zero HAT), scorer's table]

|-- Built-in WiFi --> connects to CamBox hotspot 'CC-A7F2'

|-- Built-in BLE --> pairs with scorer's PCS phone/tablet

|-- Micro-USB OTG --> connects to scorer's PCS Pro laptop (optional)

|-- USB-C charging port <-- overnight charge only (not needed on match day)

|-- Internal LiPo 3,000mAh --> powers Pi Zero 2W via PiJuice HAT (10-12hrs)

# 4. Software architecture

The system runs five distinct software services, each doing one job. They are written in Python and run as systemd services on their respective Pi devices, meaning they start automatically at boot and restart automatically if they crash.

| Service name | Runs on | Language | What it does |
| --- | --- | --- | --- |
| ble_listener.py | ScorePuck (Pi Zero 2W) | Python 3 | Advertises as a BLE peripheral, receives score strings from PCS app, forwards them to CamBox via UDP |
| usb_listener.py | ScorePuck (Pi Zero 2W) | Python 3 | Reads score strings from PCS Pro laptop via USB serial port, forwards them to CamBox via UDP |
| score_receiver.py | CamBox (Pi 5) | Python 3 | Listens for UDP packets from ScorePuck, validates HMAC signature, parses score strings into structured data |
| overlay_compositor.py | CamBox (Pi 5) | Python 3 | Updates FFmpeg score overlay in real time when new score data arrives |
| ptz_director.py | CamBox (Pi 5) | Python 3 | Sends VISCA over IP commands to PTZ camera based on match events (wicket, boundary, end of over) |

## 4.1 ScorePuck — BLE listener service (ble_listener.py)

Plain English first: The ScorePuck pretends to be a cricket scoreboard. The PCS app on the scorer’s phone looks for scoreboards to connect to. It finds the ScorePuck, connects to it, and then sends it the score after every ball — exactly like it would send to a real LED scoreboard. The ScorePuck catches each score string and immediately fires it across the WiFi to the CamBox.

### How BLE GATT works

Bluetooth Low Energy (BLE) uses a client-server model. The PCS app is the client (it writes data). The ScorePuck is the server (it receives data). The server exposes a structured data store called a GATT table, organised into Services and Characteristics.

- A Service is a logical grouping of related data — like a folder

- A Characteristic is a specific data slot inside the Service — like a file

- Each Service and Characteristic has a UUID — a unique identifier number

- PCS writes the score string into the Characteristic after every ball

### The exact UUIDs PCS uses

| Item | UUID | Type |
| --- | --- | --- |
| Scoreboard Service | 5a0d6a15-b664-4304-8530-3a0ec53e5bc1 | 128-bit custom UUID (not Bluetooth SIG standard) |
| Score Characteristic | df531f62-fc0b-40ce-81b2-32a6262ea440 | Writable — PCS writes score strings here |

### The score string format

PCS sends score data as short text strings. Each string has a 3-letter code followed by a value. Multiple strings arrive after each ball. Examples:

| String | Meaning | When sent |
| --- | --- | --- |
| BTS127/3 | Batting side Total Score = 127 runs for 3 wickets | Every ball |
| OVB4.3 | Over 4, Ball 3 (current ball in the over) | Every ball |
| WKT3 | Wicket — 3rd wicket has fallen | On wicket ball |
| COV5 | Commence Over 5 — new over starting | Start of each over |
| B1S45 | Batter 1’s Score is 45 runs | Every ball |
| B1K1 | Batter 1 is on striKe (1 = yes, 0 = no) | Every ball |
| B2S12 | Batter 2’s Score is 12 runs | Every ball |
| BTNEngland | Batting Team Name is England | Start of innings only |
| FTNAustralia | Fielding Team Name is Australia | Start of innings only |
| RRQ156 | Runs ReQuired = 156 (second innings) | Second innings only |

### BLE security — bonding

On the first pairing between the scorer's phone and the ScorePuck, they exchange long-term cryptographic keys and store each other's hardware address (MAC address). This is called bonding. On all future connections, the Pi Zero's BLE stack automatically checks that the connecting device's MAC matches the bonded device. A rogue device with a different MAC is rejected at the hardware level, before any Python code even runs.

### Key Python libraries

- bluepy or bleak — Python BLE library for Raspberry Pi

- socket — Python standard library for sending UDP packets

- hmac, hashlib — Python standard library for signing packets

## 4.2 ScorePuck — USB serial listener (usb_listener.py)

Plain English: When the scorer uses a Windows laptop running PCS Pro instead of a phone, the score data comes out of a USB port on the laptop as a serial data stream — the same standard used by printers and old-fashioned modems. The ScorePuck plugs into that USB port and reads the stream directly. The rest is identical to the Bluetooth path.

PCS Pro outputs score data on a virtual COM port at 9600 baud, 8 data bits, no parity, 1 stop bit (standard RS-232 serial settings). The Pi Zero 2W appears as a USB serial device (using its OTG port configured as a CDC gadget). The ble_listener.py and usb_listener.py services share the same forwarding logic — only the input method differs.

### Key Python libraries

- pyserial — reads from USB serial port

- socket, hmac, hashlib — identical to BLE path

## 4.3 CamBox — score receiver service (score_receiver.py)

Plain English: The CamBox listens on a specific port for incoming score messages from the ScorePuck. Each message has a digital signature on it. The CamBox checks the signature first — if it’s wrong (tampered with or from a different match), the message is thrown away. If it passes, the score is decoded and handed to the overlay service.

### Network details

- Protocol: UDP (User Datagram Protocol). Chosen over TCP because if a single ball’s score packet is lost, the next ball’s packet replaces it. No need for retransmission.

- Port: 9000 (configurable). CamBox listens on 0.0.0.0:9000 (all interfaces).

- ScorePuck sends to: 192.168.4.1:9000 (CamBox is always the WiFi hotspot gateway at this fixed IP)

### Packet structure

[ 4 bytes: match_id ] [ N bytes: score_string ] [ 32 bytes: HMAC-SHA256 signature ]

The HMAC-SHA256 signature is computed over (match_id + score_string) using the match’s pre-shared key (PSK), which is generated by CricRelay Go at match setup and stored on both the ScorePuck and the CamBox. Any packet where the signature does not verify is silently dropped. This prevents score injection attacks.

## 4.4 CamBox — overlay compositor (overlay_compositor.py)

Plain English: FFmpeg is a professional video processing tool. It takes the camera’s video stream in, draws the score graphics on top, and produces the final stream going to YouTube. The overlay_compositor service tells FFmpeg what score to show by updating a shared memory file that FFmpeg reads continuously.

### FFmpeg pipeline

ffmpeg \

-i rtsp://192.168.1.100:554/stream1 \    # Pull RTSP from PTZ camera

-vf "drawtext=textfile=/tmp/score.txt:   # Read score from shared file

fontsize=48:fontcolor=white:            # White text, 48pt

box=1:boxcolor=black@0.5:              # Semi-transparent black box behind text

x=20:y=H-th-20" \                     # Bottom-left corner

-vcodec libx264 -preset veryfast \       # H.264 encode, fast for Pi 5

-b:v 3000k \                             # 3 Mbps video bitrate

-f flv rtmp://a.rtmp.youtube.com/live2/  # Push to YouTube RTMP endpoint

[YOUTUBE_STREAM_KEY]

The overlay_compositor.py service writes the current score to /tmp/score.txt using an atomic file rename (os.replace) so FFmpeg never reads a partial write. FFmpeg re-reads the file before every frame using the reload=1 flag — the score appears in the very next encoded frame after the file is written, within 33ms maximum.

### Atomic file write — why it matters

A naive file write (open, write, close) has a race condition: if FFmpeg reads the file mid-write it gets a partial score string and displays garbage for one frame. The fix is to write to a temporary file then atomically rename it into place. On Linux, os.replace() is a single kernel rename() syscall — FFmpeg sees either the old complete file or the new complete file, never a partial.

# WRONG — race condition with FFmpeg

with open('/tmp/score.txt', 'w') as f:

f.write(score_text)

# RIGHT — atomic write, FFmpeg never sees partial content

with open('/tmp/score_new.txt', 'w') as f:

f.write(score_text)

os.replace('/tmp/score_new.txt', '/tmp/score.txt')  # single kernel syscall

### Optimised FFmpeg command — low latency pipeline

The following FFmpeg command is tuned specifically for minimum latency from camera to encoded output. Every flag is explained inline.

ffmpeg \

-fflags nobuffer \           # No input buffering — process frames as they arrive

-flags low_delay \           # Low-delay mode across entire pipeline

-rtsp_transport tcp \        # TCP for RTSP — reliable on local network

-i rtsp://192.168.1.100:554/stream1 \

-vf "drawtext=

textfile=/tmp/score.txt:   # Score file path

reload=1:                  # Re-read file EVERY frame — critical for low latency

fontsize=52:

fontcolor=white:

box=1:boxcolor=black@0.6:

x=20:y=H-th-20" \

-vcodec libx264 \

-preset ultrafast \          # Fastest encode preset — trades compression for speed

-tune zerolatency \          # Disables x264 lookahead buffers entirely

-x264-params nal-hrd=cbr:force-cfr=1 \   # Constant frame rate — stable RTMP

-b:v 3000k -minrate 3000k -maxrate 3000k \ # Constant 3Mbps bitrate

-bufsize 1500k \             # 0.5 second output buffer (3Mbps × 0.5s)

-g 60 \                      # Keyframe every 2 seconds (YouTube minimum)

-keyint_min 60 \             # Enforce consistent keyframe interval

-sc_threshold 0 \            # No scene-change keyframes (keeps interval consistent)

-f flv \                     # FLV container (required by RTMP/YouTube)

rtmp://a.rtmp.youtube.com/live2/[STREAM_KEY]

### Sponsor overlays

Sponsor logos are composited as a second FFmpeg overlay layer using the overlay filter. CricRelay Go uploads sponsor images to S3. The CamBox downloads them at match setup and switches between them on a configurable schedule (e.g. every 15 seconds during each over break).

## 4.5 CamBox — PTZ director (ptz_director.py)

Plain English: VISCA is a control language for PTZ cameras. It’s like a TV remote control sent over the network. The ptz_director service watches the incoming score events and decides what camera position to use. When a wicket falls, it zooms to the stumps. When an over ends, it pulls back to a wide shot. No camera operator needed.

### VISCA over IP basics

- VISCA packets are sent as UDP to the camera’s IP address (192.168.1.100) on port 52381

- Each command is a sequence of hex bytes. For example, zoom in: 8x 01 04 07 26 FF

- Presets are stored in camera memory (preset 1 = wide shot, preset 2 = stumps tight, etc.)

- Recall preset 1: 81 01 04 3F 02 01 FF

### Score event to camera action mapping

| PCS event received | Camera action | VISCA command type |
| --- | --- | --- |
| COV (new over) | Wide shot, both ends | Recall preset 1 |
| WKT (wicket) | Tight on stumps, hold 5 seconds | Recall preset 2, delay, recall preset 1 |
| BTS ends in 4 or 6 (boundary) | Pan toward boundary rope | Recall preset 3 (fine-tuned per ground) |
| New batter (B1 or B2 name changes) | Tighten on batting crease | Recall preset 4 |
| Between overs (COV received) | Wide establishing shot + sponsor logo | Recall preset 1 + trigger sponsor overlay |

# 5. End-to-end data flow

This section traces exactly what happens from the moment a scorer taps a ball entry in PCS to the moment the updated score is baked into the encoded video frame leaving the CamBox. The score is composited directly onto the video — it travels with the video to whatever platform receives it. There is no separate score delivery path to the viewer.

Plain English: The score is painted onto the video frames inside the CamBox. Whatever delay the streaming platform (YouTube or otherwise) adds to the video, it adds equally to the score overlay on top of it. They are always perfectly in sync. The only latency that matters operationally is the gap between scorer tap and score appearing in the encoded video — and that is what this section measures and optimises.

| Step | What happens | Optimised time |
| --- | --- | --- |
| 1 | Scorer taps ball entry in PCS app on phone/tablet | T+0ms |
| 2 | PCS app writes score string to BLE characteristic UUID df531f62... on ScorePuck. BLE connection interval set to 15ms via Connection Parameter Update request from ScorePuck at pairing time. | T+5–15ms |
| 3 | ScorePuck on_write() callback fires immediately | T+16ms |
| 4 | ScorePuck Python: compute HMAC-SHA256 signature, assemble UDP packet (match_id + score_string + 32-byte signature) | T+17ms |
| 5 | ScorePuck Python: socket.sendto() fires UDP packet to 192.168.4.1:9000 | T+17ms |
| 6 | UDP packet traverses local WiFi hotspot (no internet, no router hop) | T+18–20ms |
| 7 | CamBox score_receiver.py: recv() fires, extracts packet fields | T+20ms |
| 8 | CamBox: verify HMAC-SHA256 — if invalid, drop silently and return to recv() | T+21ms |
| 9 | CamBox: parse score string e.g. BTS127/3 → {runs: 127, wickets: 3, text: '127-3'} | T+21ms |
| 10 | CamBox: write score text to /tmp/score_new.txt then os.replace() to /tmp/score.txt (atomic) | T+22ms |
| 11 | FFmpeg drawtext filter re-reads /tmp/score.txt before next frame render (reload=1 flag) | T+22ms |
| 12 | Score graphic composited onto video frame. Maximum wait = 33ms (one frame at 30fps). Average wait = 16ms. | T+38–55ms |
| 13 | libx264 encodes frame with -preset ultrafast -tune zerolatency (no lookahead buffer) | T+55–70ms |
| 14 | Encoded frame pushed into RTMP output buffer (-bufsize 1500k = 0.5 second max) | T+70ms |
| 15 | CamBox: check if ball event triggers PTZ move (parallel with steps 10–14) | T+21ms (parallel) |
| 16 | ptz_director sends VISCA UDP preset recall to camera at 192.168.1.100:52381 (parallel) | T+22ms (parallel) |
| Total: scorer tap → score in encoded video | All stages combined, optimised pipeline | ~350–600ms |

Steps 15 and 16 run in parallel with steps 10–14 from the same parsed score event. The score overlay update and camera position move are triggered simultaneously.

## 5.1 Latency budget — before and after optimisation

| Stage | Before optimisation | After optimisation | What changed |
| --- | --- | --- | --- |
| BLE connection interval (PCS → ScorePuck) | 0–200ms, avg 100ms | 0–30ms, avg 15ms | ScorePuck requests 15ms connection interval via BLE Connection Parameter Update at pairing time |
| Python HMAC sign + UDP send (ScorePuck) | 5–10ms | 3–5ms | Pre-allocated socket, no per-packet socket creation overhead |
| WiFi UDP transit (local hop) | 1–5ms | 1–5ms | No change — already minimal |
| CamBox HMAC verify + parse + file write | 5–10ms | 2–4ms | Atomic os.replace() write, pre-compiled regex parser |
| FFmpeg frame wait (reload=1) | 0–33ms, avg 16ms | 0–33ms, avg 16ms | reload=1 already minimises this to one frame maximum |
| x264 encode latency | 16–50ms | 16–33ms | -preset ultrafast -tune zerolatency removes lookahead buffers |
| RTMP output buffer | 500ms–2,000ms | 100–500ms | -bufsize 1500k reduces buffer from default 2–5s to 0.5s |
| Total: scorer tap → encoded frame | 700ms–2,500ms | 350–600ms | ~1 second saved across all stages |

# 6. Multi-match isolation and security

Two CricRelay setups running simultaneously on adjacent pitches is a common scenario at multi-ground venues. The architecture must guarantee zero cross-contamination of score data, streams, or camera control.

## 6.1 Match setup in CricRelay Go

When a club sets up a match in the CricRelay Go web dashboard, the system automatically generates five unique credentials for that match:

| Credential | Example | Used for |
| --- | --- | --- |
| Match code | A7F2 | Human-readable ID shown to scorer. Used in device names. |
| WiFi SSID | CC-A7F2 | The name of the CamBox’s WiFi hotspot for this match |
| WiFi PSK | Xk9mW3pQ | Password for the WiFi hotspot. ScorePuck is pre-configured with this. |
| HMAC key | 32-byte random key | Signs every UDP score packet. Prevents injection attacks. |
| WiFi channel | 1 (or 6 or 11, round-robin) | Non-overlapping 2.4GHz channels reduce RF interference between adjacent matches |

A QR code containing all five credentials is displayed in CricRelay Go. The volunteer scans it with the ScorePuck’s setup app (a simple Python Flask web UI running on the ScorePuck). The CamBox receives the same credentials from CricRelay Go over HTTPS at match start.

## 6.2 BLE isolation — preventing cross-connection

### Layer 1: Unique device name

The ScorePuck advertises its BLE device name as CricRelay-A7F2 (match code appended). The scorer sees this name in the PCS External Scoreboard list. They will never accidentally connect to CricRelay-3KP9 (a different match) because the name is visibly different.

### Layer 2: BLE exclusivity

This is a fundamental BLE rule: once a peripheral connects to a central, it stops advertising entirely. No other device can see or connect to it. As soon as the scorer’s phone pairs with ScorePuck A, ScorePuck A disappears from every scan list. Match B’s scorer cannot accidentally connect to it.

### Layer 3: BLE bonding

On first connection, the scorer’s phone and the ScorePuck exchange long-term cryptographic keys (Long Term Key, Identity Resolving Key) and store each other’s Bluetooth MAC address. On all subsequent connections, the BLE stack on both devices enforces that the connecting device MAC matches the bonded device. A rogue ScorePuck with a different MAC is rejected at the hardware/firmware level before any Python code runs.

## 6.3 WiFi isolation — preventing wrong-network joins

Each CamBox broadcasts a unique SSID with a unique WPA2 pre-shared key. Even if ScorePuck B’s WiFi signal is stronger from CamBox A than from CamBox B, the WPA2 handshake will fail because the PSK is wrong. The ScorePuck firmware is configured to only join its own match’s SSID with its own match’s PSK. It will not join any other network.

Non-overlapping WiFi channels (1, 6, 11 are the three standard 2.4GHz non-overlapping channels) are assigned by CricRelay Go round-robin. This reduces RF interference even before authentication, meaning the signal from CamBox A is less likely to be received clearly by ScorePuck B in the first place.

## 6.4 Score data security — HMAC signing

Every UDP packet from ScorePuck to CamBox carries a 32-byte HMAC-SHA256 signature computed over the packet contents using the match’s pre-shared key. The CamBox verifies this signature before processing any packet. Any packet that fails verification — whether from a different match, a rogue device, or data corruption — is silently dropped.

# ScorePuck: sign the packet

import hmac, hashlib

packet_data = match_id.encode() + score_string.encode()

signature = hmac.new(psk, packet_data, hashlib.sha256).digest()

udp_packet = packet_data + signature

# CamBox: verify the packet

received_data = udp_packet[:-32]

received_sig  = udp_packet[-32:]

expected_sig  = hmac.new(psk, received_data, hashlib.sha256).digest()

if not hmac.compare_digest(received_sig, expected_sig):

continue  # drop silently — do not process

## 6.5 Security threat matrix

| Threat | Likelihood | Mitigation | Residual risk |
| --- | --- | --- | --- |
| Scorer accidentally pairs with wrong ScorePuck | Medium (multi-ground venues) | Unique device name with match code visible in PCS list | Very low |
| ScorePuck reconnects to wrong device after restart | Low | BLE bonding — MAC-locked after first pair | Negligible |
| ScorePuck joins wrong CamBox WiFi network | Low (with unique SSID/PSK) | Unique SSID + WPA2 PSK per match | Negligible |
| Rogue device intercepts BLE score data | Very low (requires proximity + knowledge) | Bonding rejects unknown MAC. Data is score strings only — not sensitive. | Low |
| Injected UDP score packets | Very low (requires WiFi access) | HMAC-SHA256 signature, dropped if invalid | Negligible |
| Score data observed in transit on WiFi | Low (WPA2 encrypted) | WPA2 WiFi encryption + HMAC adds app-layer protection | Very low |
| Two matches on same WiFi channel causing interference | Medium (busy grounds) | Non-overlapping channels assigned by CricRelay Go | Low |

# 7. Network architecture

| Interface | Device | IP address | Role |
| --- | --- | --- | --- |
| Ethernet (PoE) | CamBox Pi 5 | 192.168.1.1 (assigned by Pi as DHCP server) | Receives RTSP video and sends VISCA PTZ commands to camera |
| Camera ethernet | PTZ Camera | 192.168.1.100 (static, set in camera admin UI) | Video source and PTZ control target |
| WiFi hotspot (AP mode) | CamBox Pi 5 | 192.168.4.1 (gateway IP of hotspot) | ScorePuck connects here for score relay |
| WiFi client | ScorePuck Pi Zero 2W | 192.168.4.x (DHCP from CamBox hotspot) | Receives IP from CamBox, sends UDP score packets to 192.168.4.1:9000 |
| 4G modem (USB) | CamBox Pi 5 | Carrier-assigned public IP | Internet uplink for YouTube RTMP and CricRelay Go dashboard |

The CamBox runs two separate subnets simultaneously: 192.168.1.x for the camera (wired ethernet), and 192.168.4.x for the ScorePuck (WiFi hotspot). Linux routing handles this automatically with no special configuration needed beyond setting up the hostapd (hotspot) and dnsmasq (DHCP) services.

# 8. CricRelay OS — boot sequence and service management

CricRelay OS is a customised Raspberry Pi OS image. When powered on, the CamBox completes the following sequence automatically in under 60 seconds, with no human input required:

| Boot stage | What happens | Time from power-on |
| --- | --- | --- |
| 1. Linux boot | Pi 5 boots Raspberry Pi OS (64-bit, Lite) | 0–20 seconds |
| 2. Network interfaces | hostapd starts WiFi hotspot (CC-A7F2). dnsmasq starts DHCP for 192.168.4.x. PoE ethernet comes up at 192.168.1.1. | 20–25 seconds |
| 3. 4G modem | Quectel modem connects to carrier network. Route to internet established via usb0 interface. | 25–35 seconds |
| 4. CricRelay Go sync | CamBox fetches today’s match config (SSID, PSK, HMAC key, sponsor assets, YouTube stream key) from CricRelay Go API over HTTPS | 35–45 seconds |
| 5. Camera ping | CamBox pings 192.168.1.100. If no response within 10 seconds, LED flashes amber and retries every 5 seconds. | 45–50 seconds |
| 6. Services start | score_receiver.py, overlay_compositor.py, ptz_director.py all start as systemd services | 50 seconds |
| 7. FFmpeg starts | FFmpeg begins pulling RTSP from camera and pushing RTMP to YouTube. LED turns green. | 55–60 seconds |

If any service crashes, systemd restarts it automatically within 3 seconds. The LED turns amber during the restart and back to green when recovered. A club volunteer watching the LED has a simple status indicator without needing to touch any software.

# 9. Development setup — how to build and test locally

A developer can build and test the full CricRelay system without any of the final hardware by using a laptop, a spare Raspberry Pi 4, and a USB Bluetooth dongle. This section explains exactly how.

## 9.1 Repository structure

clubcam/

scorepuck/

ble_listener.py        # BLE GATT peripheral server

usb_listener.py        # USB serial listener (PCS Pro)

udp_sender.py          # Shared UDP send + HMAC signing

setup_wifi.py          # Writes WiFi credentials from QR code

requirements.txt       # bleak, pyserial, flask

cambox/

score_receiver.py      # UDP listener + HMAC verifier

score_parser.py        # Decodes PCS string protocol

overlay_compositor.py  # Writes /tmp/score.txt for FFmpeg

ptz_director.py        # VISCA over IP camera control

ffmpeg_runner.py       # Starts/monitors FFmpeg process

requirements.txt       # flask, requests

clubcam-go/             # Web dashboard (separate repo)

scripts/

flash_scorepuck.sh     # Burns Pi Zero 2W image with credentials

flash_cambox.sh        # Burns Pi 5 image

## 9.2 Testing without hardware

To test the full data flow on a laptop:

- Run ble_listener.py on any Linux machine with a Bluetooth adapter. Use bluetoothctl to manually send test score strings.

- Run score_receiver.py on the same machine (or a second terminal). It will receive the forwarded UDP packets.

- Use ffmpeg with a test video file as input instead of an RTSP stream: ffmpeg -re -i test_match.mp4 -vf drawtext=... -f flv rtmp://localhost/live (use nginx-rtmp locally instead of YouTube)

- Use a VISCA simulator (open source, available on GitHub) instead of a real PTZ camera to test camera control logic

## 9.3 Dependencies

| Dependency | Version | Install command | Purpose |
| --- | --- | --- | --- |
| Python | 3.11+ | Pre-installed on Pi OS | All services |
| bleak | 0.21+ | pip install bleak | BLE peripheral on ScorePuck |
| pyserial | 3.5+ | pip install pyserial | USB serial on ScorePuck |
| FFmpeg | 6.0+ | sudo apt install ffmpeg | Video encoding and overlay on CamBox |
| hostapd | 2.10+ | sudo apt install hostapd | WiFi hotspot on CamBox |
| dnsmasq | 2.89+ | sudo apt install dnsmasq | DHCP server on CamBox hotspot |
| Flask | 3.0+ | pip install flask | ScorePuck QR setup web UI |
| requests | 2.31+ | pip install requests | CricRelay Go API calls from CamBox |

# 10. Architecture decisions — rationale

Every significant technical choice in this system has a reason. This section documents what was decided and why, so future developers can understand the constraints and change decisions with full context.

## ADR-001: UDP not TCP for ScorePuck-to-CamBox score relay

| Field | Detail |
| --- | --- |
| Decision | Use UDP (connectionless) for score packet relay |
| Alternatives considered | TCP (connection-oriented), MQTT, WebSocket |
| Reason | Score data is self-contained per ball. If a packet is lost, the next ball’s packet replaces it entirely — there is no accumulated state that makes a lost packet catastrophic. TCP’s retransmission overhead on a lossy WiFi link adds unpredictable latency. UDP fires and forgets, achieving sub-15ms delivery in normal conditions. |
| Trade-off | Packet loss is not detected. Mitigated by HMAC verification catching corruption, and by score redundancy (every packet contains full current score, not just a delta). |

## ADR-002: CamBox creates WiFi hotspot, ScorePuck joins it

| Field | Detail |
| --- | --- |
| Decision | CamBox runs as WiFi AP (access point / hotspot). ScorePuck is the client. |
| Alternatives considered | External WiFi router at the ground, ScorePuck as AP, ground WiFi network |
| Reason | CamBox controls the SSID and PSK — critical for per-match isolation. No dependency on ground WiFi (which may not exist, may be congested, or may require passwords from the groundskeeper). The system must work in an empty field with no infrastructure. |
| Trade-off | CamBox WiFi is occupied as AP, so 4G modem must provide internet uplink. This is by design — the two are separate interfaces. |

## ADR-003: RTSP not HDMI for camera-to-CamBox video

| Field | Detail |
| --- | --- |
| Decision | Camera sends video as RTSP stream over ethernet. CamBox pulls RTSP with FFmpeg. |
| Alternatives considered | HDMI from camera to HDMI capture card in CamBox, USB camera |
| Reason | HDMI requires a capture card (£20–50 extra cost and another failure point), a short cable (5m max), and the camera must sit close to the encoder. RTSP over ethernet works over the same PoE cable that already carries power, runs up to 100m, and requires no additional hardware. The PTZ camera’s built-in encoder produces a clean H.264 RTSP stream at 1080p60 that FFmpeg can ingest directly. |
| Trade-off | Slight compression in the RTSP stream before FFmpeg re-encodes. At 1080p with a high-quality camera encoder this is not perceptible at grassroots streaming bitrates (3–5 Mbps). |

## ADR-004: Raspberry Pi 5 not custom ARM SoC for CamBox

| Field | Detail |
| --- | --- |
| Decision | Use Raspberry Pi 5 (4GB) as the CamBox compute module |
| Alternatives considered | Orange Pi 5, custom Android encoder box, Jetson Nano, x86 mini PC |
| Reason | Pi 5 has the largest developer community, best-maintained OS, hardware-accelerated H.264 encoding via libcamera/V4L2, GPIO for LED control, and PoE HAT availability. Software written for Pi 5 deploys identically to every unit. Orange Pi 5 has more raw compute but a fragmented software ecosystem and poor Bluetooth support. |
| Trade-off | Pi 5 costs £60 at retail vs £35 for Pi Zero or £40 for Orange Pi 5. Justified by reduced development and support cost. |

## ADR-005: Raspberry Pi Zero 2W for ScorePuck

| Field | Detail |
| --- | --- |
| Decision | Use Raspberry Pi Zero 2W as the ScorePuck compute module |
| Alternatives considered | ESP32 microcontroller, Pi 4, Nordic nRF52840 dongle, custom BLE-WiFi bridge chip |
| Reason | Pi Zero 2W runs full Linux — the same Python BLE code (bleak), signing code (hmac), and WiFi configuration as any Pi. ESP32 would require rewriting in C/MicroPython and has a more complex WiFi+BLE coexistence story. Pi Zero 2W is £15 retail, small enough to fit in the scorer’s hand, and can be reflashed with standard Pi tools. |
| Trade-off | Pi Zero 2W boots in 30–45 seconds — slower than ESP32 which boots in milliseconds. Mitigated by the ScorePuck being left powered on throughout the match day. |

## ADR-006: PiJuice Zero + LiPo 3,000mAh for ScorePuck battery

| Field | Detail |
| --- | --- |
| Decision | Use PiJuice Zero HAT with a 3,000mAh LiPo cell for ScorePuck internal battery. Target runtime: 10–12 hours. USB-C overnight charging. |
| Alternatives considered | External power bank (no cable-free), discrete TP4056 + MT3608 circuit (custom PCB required), USB-C PD power delivery hat (overkill), no battery (scorer provides power) |
| Why not external power bank | External power bank requires a cable on match day, adds to the scorer’s setup, and can be forgotten. An internal battery makes the ScorePuck genuinely cable-free on match day — the scorer’s primary operational benefit. |
| Why PiJuice Zero over discrete components | PiJuice Zero is a production-proven, Pi Zero form-factor board with certified LiPo management, GPIO-level battery monitoring (triggers low-battery LED at 15%), software-controllable power sequencing, and pass-through charging (can run Pi from USB-C while charging battery simultaneously). Building an equivalent discrete circuit adds PCB design cost, certification risk, and support burden. PiJuice Zero is ~£9 at bulk — justified by engineering time saved. |
| Why 3,000mAh | Pi Zero 2W averages ~250mA at 5V during a match. 3,000mAh LiPo at 3.7V delivers ~2,700mAh effective at 5V after boost converter losses (90% efficiency). Runtime = 2,700 / 250 = 10.8 hours. This comfortably covers a full-day T20 or limited-overs match (7–8 hours) with ~3 hours headroom. A 2,000mAh cell would cover 7 hours with no headroom — too tight. A 5,000mAh cell would not fit the enclosure without significant redesign. |
| Enclosure impact | Enclosure depth increases from 20mm to 30mm to accommodate the Pi Zero 2W + PiJuice HAT stack (18mm) plus LiPo pouch (6mm) with 6mm clearance. Footprint remains 85mm × 55mm. USB-C charging port flush-mounted on the short end. Device remains genuinely pocket-sized. |
| Trade-off | Adds ~£12–14 to ScorePuck BOM at bulk (PiJuice £9 + LiPo £4 + enclosure delta £1). Battery degrades ~20% capacity over 500 charge cycles (½3 years of weekly use). Replacement LiPo is a £4 part the club can swap themselves via a JST connector — no soldering required. |
| Battery safety | PiJuice Zero’s charge controller enforces hard voltage limits (4.2V max charge, 3.0V min discharge). LiPo selected has UL certification. Enclosure vented at battery location. Operating temperature range -10°C to +50°C covers all UK outdoor cricket conditions. |

## ADR-007: Low-latency FFmpeg pipeline — score-to-frame under 600ms

| Field | Detail |
| --- | --- |
| Decision | Implement a suite of four optimisations to reduce scorer-tap-to-encoded-frame latency from ~2,500ms worst case to ~600ms worst case. |
| Target | Score baked into encoded video within 600ms of scorer tapping ball entry in PCS. Score and video are always in sync — no separate score delivery path exists. |
| Optimisation 1: BLE connection interval | ScorePuck sends a BLE Connection Parameter Update request immediately after pairing, requesting min_interval=12 (15ms), max_interval=24 (30ms). This reduces the BLE polling window from the default 100–200ms to 15–30ms. Most modern Android and iOS phones accept this request. If the phone rejects it, the system falls back to default intervals gracefully — no failure mode. |
| Optimisation 2: Atomic file write | Score text is written to /tmp/score_new.txt and then renamed to /tmp/score.txt using os.replace(). On Linux, rename() is a single atomic kernel syscall — FFmpeg never reads a partial file. Without this, a race condition between Python’s write and FFmpeg’s read causes one-frame corrupted overlay displays. |
| Optimisation 3: FFmpeg reload=1 flag | The drawtext filter’s reload=1 parameter causes FFmpeg to re-read the score file before compositing every single frame. Without this flag, FFmpeg caches the file contents and only re-reads on a timer. With reload=1, the maximum wait from file write to overlay update is exactly one frame period (33ms at 30fps, average 16ms). |
| Optimisation 4: FFmpeg low-latency encode flags | -preset ultrafast removes x264’s lookahead buffers (which analyse future frames to improve compression but add 200–500ms of latency). -tune zerolatency disables additional buffering in x264. -bufsize 1500k reduces the RTMP output buffer to 0.5 seconds (from the default 2–5 seconds). -fflags nobuffer and -flags low_delay remove input-side buffering. Combined, these flags trade ~10–15% compression efficiency for ~1,500ms of saved pipeline latency. |
| Why not WebRTC for sub-200ms viewer latency | WebRTC would achieve ~200–500ms end-to-end viewer latency but requires a self-hosted media server (mediasoup, Janus, or similar), a custom viewer app or web player, CDN infrastructure, and SSL certificates. The engineering and infrastructure cost is disproportionate for a grassroots cricket product whose primary output is YouTube. The score is baked into the video frames — viewer latency is equal for both video and score regardless of platform. WebRTC remains a v2.0 option if the product moves to a dedicated streaming platform. |
| Trade-off | -preset ultrafast produces larger encoded frames than -preset medium for the same visual quality (approximately 15–20% larger). At a fixed 3Mbps bitrate this means slightly lower visual quality. At grassroots streaming bitrates and 1080p resolution this difference is not perceptible to viewers. If CamBox CPU load allows, -preset fast is a middle ground that recovers most compression efficiency with less latency impact than -preset medium. |

# 11. Known limitations and future work

| Limitation | Impact | Planned resolution |
| --- | --- | --- |
| BLE range 10–30m between scorer’s phone and ScorePuck | Scorer must keep phone near ScorePuck at scorer’s table | ScorePuck is designed to sit next to scorer. Range is not a problem when ScorePuck is at scorer’s table. |
| Some phones reject BLE connection interval update request | If phone rejects 15ms interval, BLE latency reverts to 100–200ms default. Total scorer-to-frame latency rises to ~700ms–1,000ms. | Graceful fallback — system remains functional, just slightly slower. All other optimisations still apply. Modern Android and iOS phones (2021+) typically accept the request. |
| WiFi hotspot is 2.4GHz only on Pi 5’s built-in WiFi | Slower than 5GHz; more congested at busy grounds | v1.2: add USB 5GHz WiFi dongle for hotspot. Score packets are tiny (≤200 bytes) so 2.4GHz is more than adequate in practice. |
| FFmpeg frame wait adds up to 33ms latency | Score file may be written 1ms after a frame render — next frame is 32ms away | Minimised by reload=1 flag (re-reads file every frame). Maximum 33ms, average 16ms. Cannot be reduced below one frame period without dropping below 30fps. |
| -preset ultrafast reduces compression efficiency ~15–20% | Slightly larger encoded frames at same bitrate = marginally lower visual quality | Acceptable at 3Mbps for 1080p grassroots streaming. Test -preset fast as a middle ground on Pi 5 — may achieve similar latency with better compression. |
| PTZ presets must be configured per ground | First setup at each ground takes 10–15 minutes | v1.2: CricRelay Go stores ground presets per venue. Second visit is automatic. |
| No automatic ball detection (cannot track cricket ball in flight) | Camera direction is score-driven, not ball-tracking | By design — ball tracking at 80mph is not achievable with a PTZ motor. Score-driven presets are the architectural solution. |
| ScorePuck LiPo degrades ~20% over 500 cycles (~3 years weekly) | Runtime reduces to ~8 hours after 3 years | Replacement LiPo JST pouch available as spare part (£4). Swap takes 2 minutes, no tools or soldering required. |
| CricHeroes (India) has no open score API | India market requires separate scoring module or CricHeroes partnership | v2.0: Approach CricHeroes for API partnership. India launch after UK/AU proven. |

# 12. Glossary

| Term | Plain English definition |
| --- | --- |
| ADR | Architecture Decision Record — a document that records a technical decision and the reasons for it |
| BLE / Bluetooth Low Energy | A low-power version of Bluetooth designed for short bursts of data, used in fitness trackers, beacons, and scoreboards |
| GATT | Generic Attribute Profile — the structured database BLE uses to organise what data a device exposes |
| Service UUID | A unique identifier number that labels a group of related data in a BLE GATT table |
| Characteristic UUID | A unique identifier for a specific data slot in a BLE Service that can be read from or written to |
| HMAC-SHA256 | A method of creating a tamper-evident digital signature on a piece of data using a shared secret key |
| UDP | User Datagram Protocol — a fast, fire-and-forget method of sending data over a network (no confirmation of delivery) |
| RTSP | Real Time Streaming Protocol — the standard way IP cameras send video over a network |
| RTMP | Real Time Messaging Protocol — the standard way streaming software sends video to YouTube and similar platforms |
| VISCA over IP | A command language for controlling PTZ cameras remotely over a network, using UDP packets |
| PoE | Power over Ethernet — technology that sends electrical power down a network cable alongside data |
| FFmpeg | A free, professional video processing tool that can ingest, transcode, overlay graphics on, and output video streams |
| Systemd service | A Linux mechanism that starts a program automatically at boot and restarts it if it crashes |
| SSID | Service Set Identifier — the name of a WiFi network (what you see when you scan for WiFi on your phone) |
| PSK | Pre-Shared Key — the password for a WPA2 WiFi network |
| WPA2 | The standard WiFi encryption protocol — protects data sent over a WiFi network from eavesdropping |
| PCS | Play Cricket Scorer — ECB’s phone/tablet scoring app for grassroots cricket in England and Wales |
| PCS Pro | Play Cricket Scorer Pro — ECB’s Windows laptop scoring software for higher-level clubs |
| HMAC key | The secret key both ScorePuck and CamBox share for signing and verifying score packets |
| IP65 / IP67 | International Protection ratings — IP65 = dust-tight + water jet resistant. IP67 = dust-tight + submersible to 1 metre. |
| DHCP | Dynamic Host Configuration Protocol — the system that automatically assigns IP addresses to devices joining a network |
| LiPo | Lithium Polymer — a rechargeable battery type. Lightweight, high energy density. Used in phones, drones, and the ScorePuck. Requires a charge controller to charge and discharge safely. |
| PiJuice Zero | A battery management HAT (add-on board) made specifically for the Raspberry Pi Zero. Handles LiPo charging, voltage conversion, battery monitoring, and power sequencing in one board. |
| HAT | Hardware Attached on Top — an add-on board that clips onto the Raspberry Pi’s 40-pin GPIO header, adding hardware capabilities without any soldering |
| JST connector | A small push-in connector commonly used for LiPo batteries. The battery plugs into the PiJuice Zero via JST. Replacement batteries are user-swappable without tools. |
| Boost converter | An electronic circuit that steps a lower voltage (3.7V from LiPo) up to a higher voltage (5V for the Pi). The PiJuice Zero contains one internally. |
| TP4056 | The charge controller chip inside the PiJuice Zero. Manages safe LiPo charging — stops at 4.2V max, cuts off at 3.0V min. |
| BLE connection interval | How often a BLE central and peripheral check in with each other to exchange data. Default is 100–200ms. CricRelay requests 15ms to reduce score delivery latency. |
| BLE Connection Parameter Update | A request from a BLE peripheral to the central (phone) to change the connection interval. The phone may accept or reject it. |
| Atomic file write | A write operation that is guaranteed to be seen as either fully complete or not at all — never partial. Achieved on Linux via os.replace() which uses a single rename() kernel syscall. |
| reload=1 (FFmpeg) | A drawtext filter flag that forces FFmpeg to re-read the score text file before compositing every single frame, rather than caching it. Essential for low-latency score updates. |
| -preset ultrafast (FFmpeg) | An x264 encoding preset that disables CPU-intensive compression optimisations in favour of speed. Produces slightly larger files but encodes each frame much faster, reducing pipeline latency. |
| -tune zerolatency (FFmpeg) | An x264 tuning option that disables the lookahead buffer — a mechanism that analyses future frames to improve compression but adds 200–500ms of encode latency. |
| -bufsize (FFmpeg) | The size of FFmpeg’s output buffer. Smaller buffer = less latency before frames are sent. Set to 1500k (0.5 seconds at 3Mbps) in CricRelay to reduce RTMP pipeline delay. |
| RTMP buffer | A buffer that holds encoded video before sending it to the streaming destination. Larger buffer = smoother stream but more latency. CricRelay uses a small buffer to prioritise score update speed. |

CricRelay ADR-001 — Every club. Every match. Live.