CricRelay Go

Starter Tier — Technical Build Document

v1.0 — May 2026

The club brings their own phone. CricRelay Go supplies everything else: the intelligence layer that adds live score overlays, 5G streaming, and stabilised video. Written so anyone can pick this up and build it.

| Field | Value |
| --- | --- |
| Tier | Starter — CricRelay Go |
| Sell price | £349 inc. VAT |
| Annual CricPass | £99/yr |
| Club provides | Their own phone (iPhone or Android) |
| We provide | CamBox, ScorePuck, 3-axis phone gimbal, 1.9m tripod, 5G modem |
| Stream quality | Up to 1080p (phone dependent) |
| Target customer | Clubs on tight budgets, junior sections, school cricket |

# 1. What does CricRelay Go do?

CricRelay Go turns a club’s own phone into a professional cricket streaming camera. The phone provides the video. CricRelay supplies the hardware and software that adds the live score on screen, connects to 5G, and stabilises the video on a tripod. It is the cheapest way to start streaming professionally.

Think of it as: the phone is the camera. CricRelay Go is the director, the score reader, and the broadcast engineer — all in a bag of hardware.

# 2. What is in the box?

| Item | What it is | What it does |
| --- | --- | --- |
| CamBox | Raspberry Pi 5 in a small weatherproof case | Pulls video from phone, adds score overlay, pushes stream to YouTube via 5G |
| ScorePuck | Raspberry Pi Zero 2W in a small branded case | Receives score data from PCS app on scorer’s device via Bluetooth, relays to CamBox over WiFi |
| 3-axis phone gimbal | Shenzhen Staranb F10-class, 3,600mAh battery, OEM branded | Holds phone steady, cancels shake and wobble with motorised stabilisation |
| 1.9m aluminium tripod | Kingjoy OEM, flip-lock legs, rubber and spike feet | Elevates phone to 1.9m for a clear pitch view over boundary hoardings |
| Quectel RM500Q-GL 5G modem | M.2 form factor, UK bands n1/n3/n28/n78 | Streams video to YouTube over 5G — works on EE, Vodafone, O2, Three |
| Carry bag | Soft branded case | Holds all components for transport to and from matches |

# 3. Hardware — component detail

## 3.1 CamBox

The CamBox is the brain. It is a Raspberry Pi 5 computer inside an IP65 weatherproof plastic box. It connects to the phone via a short USB cable, receives the live video stream, adds the score graphics on top using FFmpeg, and pushes the final stream to YouTube via the 5G modem.

| Component | Spec | Why |
| --- | --- | --- |
| Raspberry Pi 5 (4GB) | Quad-core ARM, 4GB RAM | Handles video processing and score overlay simultaneously without dropping frames |
| 5G modem | Quectel RM500Q-GL, M.2 form factor | 5G Sub-6GHz, supports all UK carriers, up to 2.5Gbps — far exceeds the 3–5Mbps needed for streaming |
| M.2 to USB 3.1 adapter | Connects M.2 modem to Pi USB port | Plug-and-play connection, no soldering |
| 5G antenna ×2 | External SMA whip antennas | Better signal than internal antennas at a ground |
| IP65 ABS enclosure | Weatherproof, pole-mount bracket | Keeps electronics dry in British weather. Mounts on tripod leg or placed in carry bag. |
| Status RGB LED | GPIO wired to Pi | Green = streaming. Amber = connecting. Red = fault. |
| Power | USB-C from phone charger or power bank (included) | Powered by a 20,000mAh power bank in the bag. Lasts 8+ hours. |

Plain English: the CamBox is like a tiny TV studio control room that fits in your hand. It takes the phone’s video, paints the score on top, and sends it to YouTube.

## 3.2 ScorePuck

The ScorePuck sits at the scorer’s table. It connects to the PCS app on the scorer’s phone via Bluetooth and relays score data to the CamBox over WiFi. It has a built-in battery so no cables are needed on match day.

| Component | Spec | Why |
| --- | --- | --- |
| Pi Zero 2W | Quad-core ARM, 512MB RAM, built-in BLE 5.0 and WiFi | Small, low power, full Linux — runs the same Python code as any Pi |
| PiJuice Zero HAT | Battery management board, stacks on Pi Zero | Manages safe charging and discharging of the LiPo battery |
| LiPo 3,000mAh | 3.7V, JST connector | 10–12 hours runtime. Charges overnight via USB-C. |
| ABS enclosure | 85mm × 55mm × 30mm, branded | Compact. Sits unobtrusively on scorer’s table. |
| Status LED | RGB, single | Blue = paired to PCS. Green = connected to CamBox. Amber = battery low. |

## 3.3 Phone gimbal

The 3-axis phone gimbal uses three motorised axes to keep the phone perfectly level and shake-free. Wind, footfall, and hand movement are all cancelled automatically. The phone sits in the gimbal’s phone clamp. The gimbal sits on top of the tripod via a standard 1/4 inch thread.

| Spec | Value |
| --- | --- |
| Axes | 3-axis (pan, tilt, roll) — all three directions of movement stabilised |
| Payload | Up to 300g — fits any phone including large Pro Max models |
| Battery | 3,600mAh built-in — 8–10 hours. Charges via USB-C. |
| Phone clamp | Adjustable, fits phones 60–90mm wide |
| Tripod mount | Standard 1/4” female thread — screws directly onto tripod head |
| OEM supplier | Shenzhen Staranb (F10/L9 class), branded CricRelay, MOQ 10 |
| Bulk cost | £22 at 50 units, £18 at 100 units |

## 3.4 Tripod

The 1.9m aluminium tripod elevates the phone and gimbal to a height where the camera can see clearly over boundary hoardings and the sight screen. It has three adjustable legs with flip-lock sections, rubber feet for grass, and removable steel spike feet for soft ground.

| Spec | Value |
| --- | --- |
| Material | Aluminium alloy (not carbon fibre at this tier — keeps cost down) |
| Maximum height | 1.9m (minimum as specified) |
| Minimum height | 0.6m (low angle shots) |
| Folded length | 72cm — fits in carry bag |
| Payload | 10kg — far exceeds gimbal + phone weight |
| Feet | Rubber feet included. Removable steel spike feet included. |
| Head | Ball head with 1/4” screw — standard camera mount |
| OEM supplier | Kingjoy Guangdong, custom CricRelay branding, MOQ 50 |

# 4. How the phone connects to the CamBox

## 4.1 Video path

The phone sends its live camera feed to the CamBox via USB. The club plugs a USB-C cable from the phone into the CamBox. The CamBox runs a UVC (USB Video Class) driver which recognises the phone as a webcam. FFmpeg pulls the video stream from the phone at 1080p30.

USB Video Class is the same standard used by all webcams. Modern iPhones and Android phones support it natively. No app required on the phone beyond enabling the camera output.

| Step | What happens |
| --- | --- |
| 1 | Scorer starts match in CricRelay Go. YouTube stream key is downloaded to CamBox automatically. |
| 2 | Club plugs phone into CamBox via USB-C cable. Phone asks permission to share camera. Tap Allow. |
| 3 | CamBox detects phone as UVC camera. FFmpeg starts pulling 1080p video. |
| 4 | FFmpeg composites score overlay onto video frames using drawtext filter. |
| 5 | Encoded H.264 stream pushed via RTMP to YouTube using 5G modem. |
| 6 | Score updates from ScorePuck update the overlay in under 600ms. |

# 5. Score system — how scores get on screen

## 5.1 The Bluetooth path (PCS phone/tablet)

The PCS app on the scorer’s device broadcasts score data via Bluetooth Low Energy after every ball. The ScorePuck, sitting at the scorer’s table, receives this data and forwards it to the CamBox over WiFi.

| Step | What happens | Time |
| --- | --- | --- |
| 1 | Scorer enters ball in PCS app | T+0ms |
| 2 | PCS writes score string to ScorePuck BLE characteristic UUID df531f62... | T+10ms |
| 3 | ScorePuck Python on_write() fires, signs UDP packet with HMAC-SHA256 | T+12ms |
| 4 | UDP packet sent to CamBox over WiFi (192.168.4.1:9000) | T+14ms |
| 5 | CamBox verifies signature, parses score string e.g. BTS127/3 | T+15ms |
| 6 | Score text written atomically to /tmp/score.txt | T+16ms |
| 7 | FFmpeg reads file on next frame render (reload=1 flag) | T+17ms |
| 8 | Updated score graphic composited onto video frame | T+50ms |
| Total | Scorer tap to score on stream | ≈350–600ms |

## 5.2 The USB path (PCS Pro laptop)

If the scorer uses PCS Pro on a Windows laptop, the ScorePuck plugs into the laptop via a short USB cable. PCS Pro sends score data via USB serial. The ScorePuck reads it and forwards it identically to the Bluetooth path. No Bluetooth needed.

# 6. Security — what happens when two matches run nearby

Two CricRelay Go setups can run on adjacent pitches without interference. Here is how isolation is guaranteed:

| Threat | Solution | Where enforced |
| --- | --- | --- |
| Scorer pairs with wrong ScorePuck | ScorePuck advertises as CricRelay-A7F2 (unique match code). Scorer taps their code only. | BLE device name |
| ScorePuck joins wrong CamBox WiFi | Each CamBox has a unique SSID and password generated by CricRelay Go. ScorePuck pre-configured with correct credentials. | WPA2 WiFi |
| Fake score packets injected | Every UDP packet signed with HMAC-SHA256 using match key. Invalid packets dropped silently. | UDP layer |
| WiFi channel interference | CricRelay Go assigns non-overlapping channels (1, 6, 11) to adjacent matches. | CamBox hostapd config |

# 7. Software stack

| Service | Runs on | What it does |
| --- | --- | --- |
| ble_listener.py | ScorePuck | Registers as BLE scoreboard, receives PCS score strings |
| usb_listener.py | ScorePuck | Reads PCS Pro score data via USB serial (alternative to BLE) |
| score_receiver.py | CamBox | Listens for UDP score packets, validates HMAC, parses strings |
| overlay_compositor.py | CamBox | Atomic write to /tmp/score.txt for FFmpeg drawtext |
| ffmpeg_runner.py | CamBox | Manages FFmpeg process: UVC video in, overlay, RTMP out |
| CricRelay Go | Cloud (AWS) | Match setup, YouTube linking, QR code generation, CricPass |

# 8. Build cost

| Component | 1 unit | 50 units | 100 units | Source |
| --- | --- | --- | --- | --- |
| Raspberry Pi 5 (4GB) | £60 | £48 | £42 | RS Components |
| Quectel RM500Q-GL 5G modem | £52 | £38 | £32 | 4GLTEMALL / Alibaba |
| M.2 USB adapter + 5G antenna ×2 | £14 | £9 | £7 | Alibaba |
| ScorePuck (Pi Zero + PiJuice + LiPo + case) | £46 | £30 | £23 | Assembled |
| 3-axis phone gimbal (OEM Staranb) | £38 | £22 | £18 | Staranb MOQ 10 |
| 1.9m aluminium tripod (Kingjoy OEM) | £28 | £16 | £13 | Kingjoy MOQ 50 |
| CamBox IP65 enclosure | £12 | £7 | £6 | Alibaba |
| 20,000mAh power bank | £22 | £14 | £11 | Alibaba OEM |
| Carry bag, cables, SD card, misc | £16 | £10 | £8 | Alibaba |
| Import duty + shipping (landed UK) | £18 | £12 | £10 | HMRC 3.5% + freight |
| Assembly + QC | £28 | £18 | £14 | UK |
| Total BOM | £182 | £124 | £102 |  |
| Sell price (inc. VAT) | £349 | £349 | £349 |  |
| Gross margin | £167 · 48% | £225 · 64% | £247 · 71% |  |

# 9. Match day setup — step by step

Total time: approximately 5 minutes.

- Night before: open CricRelay Go, create match, scan QR code with ScorePuck, charge ScorePuck and power bank overnight.

- Arrive at ground: unfold tripod, extend legs to 1.9m, lock each section.

- Attach phone gimbal to tripod head via 1/4 inch thread. Tighten finger-tight.

- Clip phone into gimbal clamp. Power on gimbal — it will level itself automatically.

- Connect phone to CamBox via USB-C cable. Phone asks to share camera — tap Allow.

- Power on CamBox via USB-C from power bank. Watch LED: amber → amber flash → blue → green.

- Give ScorePuck to scorer. Scorer powers it on, opens PCS, taps CricRelay-A7F2 in External Scoreboard list.

- ScorePuck LED turns green. Check CricRelay Go shows Stream active. You are live.

# 10. Glossary

| Term | Plain English |
| --- | --- |
| UVC | USB Video Class — the standard that lets a phone or camera share its video via USB, like a webcam |
| BLE | Bluetooth Low Energy — a low-power Bluetooth standard used for short bursts of data like score strings |
| UDP | User Datagram Protocol — a fast way to send data over WiFi without waiting for confirmation |
| HMAC | A digital signature on a data packet. Proves it came from the right device and was not tampered with. |
| FFmpeg | Free professional video software that composites the score overlay onto the video stream |
| RTMP | The protocol used to send live video to YouTube |
| 5G modem | A device that connects to the 5G mobile network to provide internet for the stream |
| PoE | Not used in Go tier (relevant to Pro and Elite only) |
| CricPass | The annual software licence that includes CricRelay Go dashboard, highlights, and support |
| Match code | A 4-character code (e.g. A7F2) that uniquely identifies a match and its WiFi and Bluetooth credentials |

CricRelay Go — Every club. Every match. Live.